AP DROUGHT MITIGATION PROJECT

- This Document contacts detailed proposal with budget for the Ground Water collectivization activity for Patch Name: Kothegili Ummadi Neeti Yajamanys Sangham.

- Detailed Pipeline Estimates and Civil Works estimate Excel sheet is also send over mail

- LFA Name	: 	APMAS

- FA name	: 	Jana Jaguruti

- District		:	Anantapur

- Mandal Name	:	Thanakalu

- GP		:	Geme Naik Thanda

- Villages		:	Mogilichatla Thanda

Ground Water Collectivization Proposal

SPMU, APDMP

Department of Agriculture, Guntur

Table of Contents

1. Context of the Patch Chosen	3

1.1. No of Plots:	4

1.2. Total Acreage:	4

2. Plot Map	5

2.1. Plots Demarcated on Google Earth	5

2.2. Pipeline Network with Legends on Google Earth	6

3. Budget Proposed Summary	7

4. CheckList	.

Annexure 1: Details of Ground water Sharing Farmers	10

Annexure 2: Bore well Discharge Measurement to determine system capacity to provide Critical Irrigation in Kharif	10

Annexure 3 : Proposed Cropping pattern	11

Annexure 4: Hydro marker survey for calculation head of pipeline & class	13

Annexure 5 : Crop water Demand for proposed Kharif crops	13

Annexure 6: Bore well Pooling Pipe Line Network detailed estimation	15

Annexure 7 : Civil Works Estimate	16

- Context of the Patch Chosen.

Mogilichatla Thanda village is located 114 km from Anantpur district. The agricultural dependency of this village is on rainfed agriculture. Farmers of this village is facing frequent crop loss due to rainfall uncertainty and long dry spells. WASSAN team visited the field on 22nd September 2018 along with LFA team for feasibility study and explained the rules and regulation of groundwater sharing norms in the Mogilichatla Thanda village meeting in front of farmers group. Mogilichatla Thanda village farmers identified perennial bore wells and formed a group of water sharing farmers. They have requested to FPO for creation of the critical irrigation facility through Groundwater Collectivisation & Sharing. The group was agreed for all rules and regulations of groundwater management and water sharing norms .The FA Jana Jaagriti identified the group area and did detailed surveys with support of WASSAN LTA team. WASSAN technical team prepared water sharing pipe line network design and estimation based on the primary data provided by the FA. The proposal shared with FPO and water sharing group by Jana Jaagriti FA team and taken consent of the community. They have further submitting to you for approval plan and budget for taking of proposed water sharing network.

The proposal will be benefit to 26 farmers, 18 farmers is rainfed and 8 are the bore-well farmers out of the total. The proposed 64 acres of water sharing farm land, 47.5 acres of land is rainfed and 16.5 acres of land is irrigated. The bore-wells are within the patch of land. By creating water sharing network farmers will over comes from small to medium droughts spells by accessing with assured protective irrigation from water sharing pipe line network in 65 acres of rainfed crop like Groundnut and red gram. Rainfed farmers will be secured crop cultivation investment and get additional benefit of Rs.3000/ to 5000/- per acre with improved yield with comparative to non-protective irrigation crops. Bore well farmers also get the benefit of protective irrigation to their rainfed crops. Apart from the crop security, they are securing their bore wells defunct due to new bore wells drilling with mutually agreed norm of ban on new bore-well drilling by water sharing farmers.

  - No of Plots:

The total numbers of patches are 26.

  - Total Acreage:

The total farms area is 64 acres, the rainfed land is 47.5 acres and irrigated land is 16.50 acres.

Table 1: Farmers Summary Table

| Description | Number of Farmer | Area (acre) | Remarks |
| --- | --- | --- | --- |
| Total | 26 | 64 |  |
| Bore well ownership |  |  |  |
| Number of Bore well Farmers | 8 | 22 | All the 8 bore-wells has good yield. |
| Number of Non Bore well Farmers | 18 | 42 |  |
| Category wise |  |  |  |
| Small and Marginal Farmers | 24 | 54 |  |
| Others | 2 | 10 |  |
| Gender |  |  |  |
| Male Farmers | 28 | 42 |  |
| Women Farmers | 8 | 22 |  |
| Caste Wise |  |  |  |
| SC / ST Farmers | 25 | 62 |  |
| Others Farmers | 1 | 2 |  |

- Plot Map

2.1. Plots Demarcated on Google Earth.

  - Pipeline Network with Legends on Google Earth.

- Budget Proposed Summary.

| Sl. No. | Work details | Estimate budget (Rs.) | Source of fund |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | DoA Share (Rs.) | Farmer Share (Rs.) | % over DoA |
| 1 | Civil work | 397570 | 294669 | 102900 | 34.92 |
| 2.1 | Pipe line work- PVC pipes | 597616 | 597616 | 0 | 0.00 |
| 2.2 | Pipe line work- Pipe line installation and fittings | 458916 | 331796 | 127120 | 38.31 |
| 3 | Sprinkler Set | 267830 | 267830 | 0 | 0.00 |
| 4 | Construction of compost pit | 251189 | 152277 | 98912 | 39.38 |
| 5 | Compost preparation (3 cycles) | 1041867 | 520934 | 520934 | 50.00 |
|  | Total | 3014989 | 2165122 | 849866 | 39.25 |
|  | % of each stake holder | 100.00 | 71.81 | 28.19 |  |

- Cost Benefits

| Sl. No | Benefits | Total Area (acres) / No of Bore wells | Annual additional Income per acre Amount (Rs.) | Total Annual additional Income per acre Amount (Rs.) | Total water sharing agreement period in years | Total Amount (Rs.) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Additional income yield | 64 | 4000 | 256000 | 10 | 2560000 | Based on current experience of pilot site of RRA |
| 2 | Protecting crop loss and cost of cultivation in major droughts | 64 | 4000 | 256000 | 4 | 1024000 | Based drought trend |
| 3 | Borewells farmers irrigated crops. | 10 | 7500 | 75000 | 10 | 750000 | Based on current experience of pilot site of RRA |
| 4 | Protecting borewells from defunct | 3 | 100000 | 300000 | 3 | 900000 | Based on current experience of bore well failure |
|  | Total |  |  | 887000 |  | 5234000 |  |
|  | Total Investment |  |  |  |  | 3014989 |  |
|  | Cost Benefits Ratio |  |  |  |  | 1.73599323 |  |
|  | Payback Period (Annual) |  |  |  |  | 753.747178 |  |

- Checklist

| Sl. No. | Particulars | Yes /No | Remarks |
| --- | --- | --- | --- |
| 1 | Pipeline estimation | Yes | Attached |
| 2 | Civil work estimation | Yes | Attached |
| 3 | Pipeline plan | Yes | Attached |
| 4 | Location plan (Highlighter map of Cadastral) | Yes | Attached |
| 5 | Farmers details | Yes | Attached |
| 6 | Bore well water discharge details | Yes | Attached |
| 7 | Crop water demand estimation | Yes | Attached |
| 8 | Resolution of community on agreement of water sharing | No | Attached |
| 9 | Agreement document on bond paper | No | Attached |

Annexure 1: Details of Groundwater sharing farmers:

Annexure 2: Bore well discharge measurement to determine system capacity to provide Critical Irrigation for Kharif.

Annexure 3: Proposed cropping pattern.

Annexure 4: Ground level cross section map of pipe line alignment for calculation head of pipeline & class.

Annexure 5: Crop water Demand for proposed Kharif crops.

| Sl. No. | Description | Units | Value |
| --- | --- | --- | --- |
| a. | Total Discharge from all bore wells in patch in a day* | Discharge in cum/6hrs |  |
| b. | Total Number of Days requiring Critical Irrigation** | Day |  |
| c. | Total Discharge of no of days of Critical irrigation | cum | (b.) * 10 days |
| d. | One critical irrigation required water quantity per acre*** | Cum/acre | 202.4 |
| e. | Capacity of System Irrigation for the 15 days | acres | (c.) / (d.) |

*Day = average 6hrs usage    ** usually taken 10 days *** 202.40 cubic metre approx. is required in one Critical Irrigation in one acre.

**** Main pipe design with 10LPS carrying capacity at 0.75 meter/sec velocity and maximum head of 5 meter per 1000 meter long distance. Opt PVC pipe dia. 140mm and 4kg/cm^2 pressure capacity.

Annexure 6: Bore well Pooling Pipe Line Network detailed estimation.

**Please double click anywhere on the above sheet to see the pipeline estimation.

Annexure 7: Civil Works Estimate.

Group account details.

Agreement document on bond paper.

Resolution of community on agreement of water sharing.