### Methods for measuring soil moisture

There are many methods for measuring soil moisture in soil. They range from simple to complex, from cheap to expensive, and from low to high in terms of labour requirements. As the focus of this conference is on measuring soil moisture at farm level by small-scale producers, only methods that are inexpensive and simple to use are presented.

Gravimetric method

The gravimetric method is simple and reliable and is the standard against which all other methods are compared. Its chief drawback is that it is somewhat labour-intensive. Another disadvantage is that it requires more than 24 hours to yield a result. The method involves taking a sample of soil, placing it in a container of known weight, weighing the container with the moist soil, drying the sample in an oven set at 105 °C for 24 hours, and then weighing the container with the sample again. The difference in weights before and after drying equals the weight of the water. The difference in weights of the container containing the dried soil and the container weight equals the weight of the dry soil. The mass wetness content of the soil is then calculated by dividing the weight of the water by the weight of the dry soil, and then converting it to a percentage by multiplying the result by 100. For example, a can that weighs 30 g is filled with moist soil, and the weight of the can plus the moist soil is 145 g. After drying the soil in the oven for 24 hours, the weight of the can plus the dry soil is 125 g. Therefore, the weight of water is 20 g (145 - 125 g) and the weight of the dry soil is 95 g (125 - 30 g). The mass wetness percentage is: (20/95) x 100 = 21.05 percent.

Under field conditions, soil moisture samples should be determined for each 30-60-cm layer of the rooting depth in order to obtain a representative picture of the soil moisture status. As major crops in rainfed areas usually extract water to a depth of at least 120 cm, then about 5 or 6 samples are required for each site measured, and several sites are usually suggested to characterize a field. Therefore, this method requires considerable time, but the results are accurate and the equipment required is minimal and inexpensive. Any type of oven can be used as long as the temperature can be maintained at about 105 °C.

SOIL MOISTURE ESTIMATION

A) Direct methods:

I) Gravimetric methods: In the gravimetric method, basic measurement of soil moisture is made on soil samples of known weight or volume. Soil sample from the desired depths are collected with a soil auger. Soil sample are taken from desired depth at several locations of each soil type. They are collected in air tight aluminum containers. The soil samples are weighed and they are dried in an oven at 105 oC for about 24 hours until all the moisture is driven off. After removing from oven, they are cooled slowly to room temperature and weighed again. the difference in weight is amount of moisture in the soil. The moisture content in the soil is calculated by the following formula:-

Moisture content           Wet weight –Dry weight
On weight basis    = ----------------------------- X 100
                                           Dry weight

PROBLEMS: Wet weight of a soil sample with can is 210 gms and weight with can is 180 gms weight of empty container is 40 gms calculated moisture content of soils sample?

Solution:
     Weight of wet soil sample = wet weight – weight of empty can
                                                      = 210-40
                                                      = 170
             Dry weight of soil sample = Dry weight – weight of can
                                                      =180-40
                                                      =140

Wet weight of soil- Dry weight of soil
    Moisture content (%) = --------------------------------------- X 100
                                                Dry weight of soil

170-140
                                                     = ----------------------------- X100
                                                                  140
                                                                                                                                                   				      30
                                                     =   ------   X 100

140

= 21.4%

Source : Dr. S. Riazuddin Ahmed Principal Scientist (SS) & Head Retired ANGRAU, Rajendra Nagar, Hyderabad.