COMPILATION OF NIGHT SHELTER MODELS

1. Referred  by Dr. G .Sreenivas Reddy , APD Kadapa

Description / remarks : it is taken from book , do not have cost norms , but useful in field conditions.

2.Referred by Dr.Gopal Reddy , LS , APMASS

Description /remarks :

Practically  : 1meter = 3.2 feet =39.3 inches

i.e. 8meters length , 6 -7 meters width

pillars used  are :

3 meters height = 2

2meters height = 4

area covered = roughly 48 sq.mts.

it can accommodate  = 35-40 sheep

This is the 2nd model . Both the sheds are same but difference is in the open area covered.

Traditionally used  protective shelter for lambs and kids

3rd model

Details of sheep shelter of Sri sanjeevappa

Village : vanaganapalli

Mandal: parigi mandal

District : Anatapuramu dt 
Mobile no: 9704913490

Measurements :

Total area = 30 ft * 30 ft ( length * breadth )
fenced with =  5ft  height G.I. mesh  all aroud i.e. 120 feet fencing area

a open shed inside with measurements = length * breadth =  30 ft * 18 ft  dimensions

Roofing material =  22 gi sheets of 3 by 10 ft size  ,

In the central row =  12 ft  height Stone pillars = 5 pillars

At sides = six ft height pillars = 10

Six rows of 1.5 inch G.I. horizontal pipe and 10 ft length 2 inch pipes 12 for support are usedfo

it can accommodate 100 sheep  .

During d night sheep will freely move around in open pen and during rain they take shelter under d shed  .

the entire structure costed him  around rs 40000=00

FRONT VIEW

REAR VIEW

SIDE VIEW

OVER VIEW

MATERIAL USED :

1.CIRCULAR PIPE AS SUPPORTING PIPES

2.SQUARE PIPE

3.G.I.SHEET

STRUCTURE OF THE SHED

| S.NO | CATEGORY | UNITS | COST/ UNIT | REQUIRED QUANTITY IN NUMBER | COST | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CENTRAL POLES | FEET | 1000 RS /20 FEET | 5 | 3000 | 1.6 MM GAUGE , 12 FEET HEIGHT |
| 2. | G.I. SHEETS | FEET | RS 100-120 / FEET | 22 | 22,000 |  |
| 3. | SUPPORTING PIPES | FEET |  | 10 | 3850 | 1.2 MM GAUGE ,1.5 INCH PIPE ,10 FEET IN LENTH |
| 4. | RIDGE | FEET | 20 RS/FT | 1 | 640 | 32 FEET IN LENGTH |
| 5. | HORIZONTAL PIPES /SQUARE PIPE | FEET | 650 RS/10.5 LENGTH | 7 | 6825 | 1.5 INCH PIPE , 1.2 MM GAUGE |
| 6. | SIDE POLES / STONE PILLARS |  |  | 10 | 1400 |  |
| 7. | J-BOLTS AND OTHERS |  |  |  | 1500 |  |
|  | TOTAL |  |  |  | 39,215 |  |

THIS IS EXCLUDING THE LABOUR AND MATERIALS LIKE BRICKS AND CEMENT

1.FENCING = 120 FEET AREA SHOULD BE COVERED , 5 FEET HEIGHT MESH IS USED =12,000

2.STONE PILLARS FOR SUPPORT = 3000

TOTAL = 15,000

I.E. = 40,000+15000 = 55,000

ANOTHER MODEL

DESCRIPTION :

26*14 FEET SHED .IT CAN ACCOMMODATE 40-50 ADULT SHEEP.THREE SIDES ARE COVERED WITH KADAPA SLABS AND ONE SIDE WITH MESH. APPROXIMATE COST IS PROVIDED BELOW

| S.NO | CATEGORY | UNITS | COST/ UNIT | REQUIRED QUANTITY IN NUMBER | COST | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | CENTRAL POLES | FEET | 1000 RS /LENGTH (2.5 LENGTHS ARE REQUIRED) | 4 | 2500 | 1.6 MM GAUGE , 12 FEET HEIGHT |
| 2. | G.I. SHEETS | FEET | RS 800/ ONE SHEET | 18 | 14400 | 8 FEET *3FEET |
| 3. | SUPPORTING PIPES | FEET | 650 RS /3 | 8 | 1950 | 1.2 MM GAUGE ,1.5 INCH PIPE ,8 FEET IN LENTH |
| 4. | RIDGE | FEET | 20 RS/FT | 1 | 520 | 26 FEET IN LENGTH |
| 5. | HORIZONTAL PIPES /SQUARE PIPE | FEET | 600 RS/20 FEET LENGTH | 5 | 4800 | 1.5 INCH PIPE , 1.2 MM GAUGE 30 FEET LENGTH |
| 6. | SIDE POLES / STONE PILLARS |  |  | 10 | 1400 |  |
| 7. | MESH | FEET | RS 20 /SQ.FT | 26 | 2600 | FEET |
| 8. | SIDE STONES | FEET |  |  | 1600 |  |
| 9. | J-BOLTS ,BRICKS,CEMENT AND LABOUR |  |  |  | 5000 |  |
|  | TOTAL |  |  |  | 38,870 |  |

This can accommodate 40-50 sheep.

Another model for Sheep NS

Dear all this is a night shelter for sheep at Nallagutlapalli N P KUNTA mandal Anatapuramu  .for the information of all the approximate value is as follows

- Size = 30* 12 feet

- one side covered with 5ft ht mesh

- 2 sides with Kadapa slabs

- 1 side wall  the cost is  21 Kadapa slabs @rs 240 each  ,

- cost of mesh  5 by 35 ft @rs 2500

- top sheet Rs 500  so d total cost excluding labor n poles  is RS 8050  .

- this can accommodate approximately 50 sheep as it is open

POULTRY SHELTER ESTIMATION :

REFERRED BY Dr.S.RADHIKA , LS , RASS.

SHED SIZE = 70 FEET *27 FEET BIRD CAPACITY = 2000 BIRDS

| S.NO | MATERIAL | COST/UNIT | REQUIRED UNITS | TOTAL COST | REMARKS |
| --- | --- | --- | --- | --- | --- |
| 1 | FOUNDATION STONE | 1500 /LOAD | 3 | 4500 |  |
| 2 | BRICKS | 13000/LOAD | 4 | 52000 |  |
| 3 | CEMENT | 340/BAG | 35 | 11900 |  |
| 4 | SAND | 2000/LOAD | 4 | 8000 |  |
| 5 | STONE PILLARS LARGE | 1200/PILLAR | 6 | 7200 |  |
| 6 | STONE PILLARS MEDIUM | 600/PILLAR | 12 | 7200 |  |
| 7 | IRON RODS LARGE | 1300 /ROD | 12 | 15600 |  |
| 8 | IRON RODS MEDIUM | 700 /ROD | 20 | 14000 | ONE LINE 35 RODS ,TOTAL 6 LINES |
| 9 | ASBESTOS SHEETS ISI MARK | 1150 /SHEET | 100 | 115000 | ONE LINE 25 SHEETS. TOTAL 4 LINES |
| 10 | IRON MESH | 80/ KG | 70 | 5600 | ONE SIDE 35 KG |
| 11 | GUTTER TILES | 50 /TILE | 10 | 500 |  |
| 12 | LABOUR COST |  |  | 20000 |  |
|  | TOTAL |  |  | 2,61,000 |  |

POULTRY NIGHT SHELTER

REFERRED BY Dr.ESHWARAIAH ,LS ,ARANYA

It is costing about rs2400 excluding his own material and labour.

Sheep shelter designs

Referred by Dr.P.SIVAKUMAR ,APD ,KURNOOL.